| POWERTRAIN |  |  |  |
| --- | --- | --- | --- |
| Engine Type | 3.5-liter, SOHC VTEC 60-degree V-6 |  |  |
| Horsepower | 253 hp @ 5800 rpm SAE Net (Rev 8/04) |  |  |
| Torque | 250 lbs-ft at 3500 rpm SAE Net (Rev 8/04) |  |  |
| Redline | 6300rpm |  |  |
| Bore and Stroke | 3.5 in x 3.66 in (89mm x 93 mm) |  |  |
| Displacement | 212 cu in (3471 cc) |  |  |
| Compression Ratio | 10.0:1 |  |  |
| Induction System | Multi-Point Fuel Injection |  |  |
| Valvetrain | Variable Valve Timing and Lift Electronic Control (VTEC), 4 valves per cylinder, belt-driven, single overhead camshaft |  |  |
| Engine Block | Aluminum alloy |  |  |
| Emission Control | ULEV-2 emissions |  |  |
| Throttle Ignition System | Drive-By-Wire Throttle System Direct ignition system with knock control |  |  |
| Alternator | 130 amp. Max |  |  |
| Battery | 12V, maintenance free |  |  |
| Recommended Fuel | Premium Unleaded |  |  |
| Layout | Transverse mounted, front engine, four wheel drive |  |  |
| Drivetrain | VTM-4 Variable Torque Management 4-wheel drive |  |  |
| Transmission | Electronically controlled, 5-speed automatic with Grade Logic Control |  |  |
|  | Ratios (:1) | 1st 2nd 3rd 4th 5th Reverse Final | 2.693 1.566 1.023 0.729 0.531 1.889 4.375 |

| CHASSIS |  |  |  |
| --- | --- | --- | --- |
| Body Type | All-steel unit body |  |  |
| Front Suspension | Independent strut with lower control arm |  |  |
| Rear Suspension | Independent multi-link with trailing arm |  |  |
| Shock Absorbers | Gas-pressurized |  |  |
| Stabilizer Bars Front | 23 mm diameter |  |  |
| Rear | 20 mm diameter |  |  |
| Steering Type | Variable power-assisted, rack-and-pinion |  |  |
| Steering Ratio | 16.9 : 1 |  |  |
| Steering Wheel Turns (lock to lock) | 3.22 |  |  |
| Turning Circle (curb to curb) | 38ft (11.6m) |  |  |
| Wheels | 17 x 6.5 JJ cast-aluminum alloy |  |  |
| Tires | P235/65R17 all-season |  |  |
| Braking System | 4-wheel disc brakes with 4-channel Anti-lock Braking System (ABS) and Electronic Brake Distribution (EBD) |  |  |
| Front Discs | Ventilated 11.8 in (300 mm) diameter; 1.1 in (28 mm) rotor thickness |  |  |
| Rear Discs | Solid 12.3 in (313 mm) diameter; 0.4 in (11 mm) rotor thickness |  |  |
| Total Swept Area | Front: 63.1 sq in (407.2 cm2) Rear: 50.4 sq in (325.0 cm2) |  |  |
| Anti-lock Braking System (ABS) | 4-channel system with 4-wheel speed sensors and electric/hydraulic control |  |  |
| CAPACITIES |  |  |  |
| Crankcase | 5.3 US qt (5.0 L) |  |  |
| Cooling System | 2.4 US gallons (9.0 L) |  |  |
| Fuel tank | 20.4 US gallons (77.0 L) |  |  |

| CAPACITIES (continued) |  |  |  |
| --- | --- | --- | --- |
| Volumes | Passenger |  | 142.9 cu ft |
|  | Cargo with 2nd and 3rd row seats down |  | 81.5 cu ft |
|  | Cargo with 3rd row seats down |  | 44.4 cu ft |
|  | Cargo behind 3rd row seats |  | 14.8 cu ft |

| FUEL ECONOMY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EPA Fuel Mileage – City/Highway | 17 / 23 MPG |  |  |  |  |  |  |  |
| EXTERIOR DIMENSIONS |  |  |  |  |  |  |  |  |
| Wheelbase | 106.3 in | (2700 mm) |  |  |  |  |  |  |
| Track, front | 66.3 in | (1685 mm) |  |  |  |  |  |  |
| Track, rear | 66.5 in | (1690 mm) |  |  |  |  |  |  |
| Overall Length | 188.7 in | (4789 mm) |  |  |  |  |  |  |
| Overall Width | 77.0 in | (1955 mm) |  |  |  |  |  |  |
| Overall Height | 68.7 in | (1744 mm) |  |  |  |  |  |  |
| Minimum Ground Clearance | 8.0 in | (203 mm) |  |  |  |  |  |  |
| Approach/Departure Angle | Approach–28 degrees |  | Departure–21 degrees Breakover –19.4 degrees |  |  |  |  |  |
| WEIGHTS |  |  |  |  |  |  |  |  |
| Curb Weight | 4471 lbs (2028 kg) | Touring Package – 4504 lbs (2043 kg) |  |  |  |  |  |  |
| W/Acura Navigation System | 4515 lbs (2048kg) |  |  |  |  |  |  |  |
| W/Rear Entertainment System | 4531 lbs (2055kg) |  |  |  |  |  |  |  |
| Weight Distribution (% front / rear) 56/44 |  |  |  |  |  |  |  |  |
| TOWING |  |  |  |  |  |  |  |  |
| Maximum Towing Capacity | 4500 lbs boat/3500 lbs trailer |  |  |  |  |  |  |  |
| Maximum Tongue Weight | 450 lbs |  |  |  |  |  |  |  |

| INTERIOR DIMENSIONS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Front | Head Room | 38.4 in | (976 mm) |  |
|  | Leg Room | 41.5 in | (1055 mm) |  |
|  | Hip Room | 56.9 in | (1446 mm) |  |
|  | Shoulder Room | 61.2 in | (1555 mm) |  |
| 2nd Row Rear | Head Room | 39.0 in | (990 mm) |  |
|  | Leg Room | 37.8 in | (959 mm) |  |
|  | Hip Room | 56.3 in | (1431 mm) |  |
|  | Shoulder Room | 61.1 in | (1551 mm) |  |
| 3rd Row Rear | Head Room | 36.1 in | (918 mm) |  |
|  | Leg Room | 29.3 in | (745 mm) |  |
|  | Hip Room | 48.5 in | (1232 mm) |  |
|  | Shoulder Room | 58.6 in | (1489 mm) |  |
| WARRANTY |  |  |  |  |
| Vehicle | 4-year / 50,000-mile limited warranty |  |  |  |
| Powertrain | 6-year / 70,000-mile limited warranty |  |  |  |
| Outer Body Rust-Through | 5-year / unlimited-mile limited warranty |  |  |  |